○ Ansh Andotra

○ iot-052

○ Course Name: Internet of Things (IoT)

○ IOT ASSIGNMENT 3

Q35. REST API – Weather Data Fetcher

Features

- Connects ESP32 to Wi-Fi

- Reads local temperature and humidity from DHT11

- Fetches live weather from OpenWeatherMap API

- Displays:

  - City Name

  - API Temperature

  - Local DHT11 Temperature

  - Temperature Difference

  - API Humidity

  - Local Humidity

  - Humidity Difference

  - Weather Description

- Stores API key separately in config.h

- Keeps config.h out of Git using .gitignore

config.h

#ifndef CONFIG_H

#define CONFIG_H

// WiFi Credentials

const char* WIFI_SSID = "YOUR_WIFI_NAME";

const char* WIFI_PASSWORD = "YOUR_WIFI_PASSWORD";

const char* API_KEY = "YOUR_OPENWEATHERMAP_API_KEY";

City

const char* CITY = "Jammu";

Country Code

const char* COUNTRY = "IN";

#endif

2. WeatherProject.ino

#include <WiFi.h>

#include <HTTPClient.h>

#include <ArduinoJson.h>

#include <DHT.h>

#include "config.h"

// DHT11 Settings

#define DHTPIN 4

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

String serverName =

"http://api.openweathermap.org/data/2.5/weather?q=" +

String(CITY) + "," + String(COUNTRY) +

"&appid=" + String(API_KEY) +

"&units=metric";

void setup() {

Serial.begin(115200);

dht.begin();

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

Serial.print("Connecting");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi Connected!");

}

void loop() {

float localTemp = dht.readTemperature();

float localHumidity = dht.readHumidity();

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

http.begin(serverName);

int httpResponseCode = http.GET();

if (httpResponseCode > 0) {

String payload = http.getString();

DynamicJsonDocument doc(2048);

deserializeJson(doc, payload);

String cityName = doc["name"];

float apiTemp = doc["main"]["temp"];

float apiHumidity = doc["main"]["humidity"];

String description =

doc["weather"][0]["description"];

Serial.println("--------------------------------");

Serial.print("City : ");

Serial.println(cityName);

Serial.print("Weather : ");

Serial.println(description);

Serial.println();

Serial.print("API Temperature : ");

Serial.print(apiTemp);

Serial.println(" °C");

Serial.print("Local DHT11 Temperature : ");

Serial.print(localTemp);

Serial.println(" °C");

Serial.print("Temperature Difference : ");

Serial.print(localTemp - apiTemp);

Serial.println(" °C");

Serial.println();

Serial.print("API Humidity : ");

Serial.print(apiHumidity);

Serial.println(" %");

Serial.print("Local DHT11 Humidity : ");

Serial.print(localHumidity);

Serial.println(" %");

Serial.print("Humidity Difference : ");

Serial.print(localHumidity - apiHumidity);

Serial.println(" %");

Serial.println("--------------------------------");

}

else {

Serial.print("HTTP Error : ");

Serial.println(httpResponseCode);

}

http.end();

}

else {

Serial.println("WiFi Disconnected");

}

delay(30000);

}

3. Required Libraries

Install these libraries from the Arduino Library Manager:

- WiFi (ESP32)

- HTTPClient

- ArduinoJson (latest version)

- DHT sensor library by Adafruit

- Adafruit Unified Sensor

4. Serial Monitor Output

Example:

Connecting....
WiFi Connected!


City : Jammu
Weather : scattered clouds

API Temperature : 31.4 °C
Local DHT11 Temperature : 30.8 °C
Temperature Difference : -0.6 °C

API Humidity : 72 %
Local DHT11 Humidity : 69 %
Humidity Difference : -3 %

Q36. MQTT vs HTTP Comparison

Comparison of MQTT and HTTP

| Parameter | MQTT | HTTP |
| --- | --- | --- |
| Architecture Pattern | Publish–Subscribe architecture using a broker. Devices publish messages to topics, and subscribers receive updates from the broker. | Client–Server architecture where a client sends requests directly to a server, which responds with data. |
| Data Transfer Model | Asynchronous and event-driven. Data is transmitted only when needed, making communication efficient. | Synchronous request–response model. The client must initiate every communication, even to check for new data. |
| Power Consumption | Low power consumption due to small packet size, persistent connections, and minimal overhead. Suitable for battery-powered IoT devices. | Higher power consumption because each request requires establishing or maintaining communication with larger protocol overhead. |
| Latency | Very low latency because messages are delivered immediately through the broker using lightweight packets. | Higher latency due to repeated request-response cycles and larger protocol headers. |
| Use Case Suitability for IoT | Ideal for IoT applications such as smart agriculture, smart homes, industrial monitoring, healthcare devices, and sensor networks. | Better suited for web applications, REST APIs, dashboards, firmware downloads, and occasional device configuration. |
| Security Considerations | Supports SSL/TLS encryption, username/password authentication, certificates, Access Control Lists (ACLs), and secure brokers. Security depends heavily on broker configuration. | Supports HTTPS (TLS/SSL), authentication methods (Basic, OAuth, JWT), firewalls, and mature web security mechanisms. Easier to integrate with existing web services. |

Analysis

MQTT Advantages

- Lightweight protocol with very small packet size.

- Requires low bandwidth.

- Consumes less battery power.

- Supports real-time communication.

- Efficient for thousands of connected devices.

- Offers Quality of Service (QoS) levels (0, 1, and 2) for reliable message delivery.

MQTT Disadvantages

- Requires an MQTT broker (e.g., Mosquitto or EMQX).

- Slightly more complex infrastructure than HTTP.

HTTP Advantages

- Simple and widely supported.

- Easy integration with REST APIs and web applications.

- Excellent for configuration interfaces and cloud services.

HTTP Disadvantages

- Larger protocol overhead.

- Higher latency.

- Greater bandwidth usage.

- More power consumption.

- Not ideal for continuous sensor communication.

Choice for a 1000-Node Smart Agriculture Deployment

For a 1000-node smart agriculture deployment, I would choose MQTT.

Reasons

- Scalability

  - MQTT is designed to efficiently support thousands of connected devices through a broker.

- Low Power Consumption

  - Agricultural sensor nodes are often battery- or solar-powered. MQTT minimizes energy usage by transmitting only necessary data.

- Efficient Bandwidth Usage

  - Farms may rely on limited wireless networks (Wi-Fi, LoRa gateways, or cellular). MQTT's lightweight messages reduce network congestion.

- Low Latency

  - Critical events such as low soil moisture, irrigation control, or weather alerts can be delivered almost instantly.

- Reliable Communication

  - MQTT's QoS levels ensure important sensor data is delivered even if network connectivity is intermittent.

- Event-Driven Operation

  - Sensors can publish data only when measurements change significantly, reducing unnecessary transmissions and extending device battery life.

Conclusion

While HTTP is well suited for web applications and REST APIs, MQTT is the better choice for a large-scale IoT deployment such as a 1000-node smart agriculture system. Its publish–subscribe architecture, lightweight communication, low power consumption, efficient bandwidth usage, and reliable message delivery make it significantly more suitable for managing a large network of distributed sensors and actuators.

Q37. Wi-Fi Security for IoT

The rapid growth of Internet of Things (IoT) devices has increased the importance of securing Wi-Fi communications. One common attack vector is the exploitation of WPA2 vulnerabilities, such as the KRACK (Key Reinstallation Attack), which can allow attackers to intercept or manipulate wireless traffic if devices are not updated with security patches. Another significant threat is the Evil Twin attack, where an attacker creates a fake Wi-Fi access point that mimics a legitimate network. Unsuspecting IoT devices may connect to the rogue access point, enabling attackers to capture sensitive data, inject malicious commands, or perform man-in-the-middle attacks.

To secure ESP32 Wi-Fi connections, developers should adopt modern security practices. Using WPA3 instead of WPA2 provides stronger encryption and protection against offline password-guessing attacks. When communicating with cloud services, TLS/SSL encryption should always be enabled. Additionally, certificate pinning helps verify that the ESP32 connects only to trusted servers by validating a known server certificate or public key, reducing the risk of man-in-the-middle attacks. Regular firmware updates, strong unique passwords, secure boot, and flash encryption further improve device security.

Hardcoding Wi-Fi passwords or API keys directly into IoT source code is dangerous because these credentials can be extracted if the firmware is reverse-engineered or if the source code is accidentally published in a public repository. Once exposed, attackers can gain unauthorized access to networks, compromise connected devices, or steal sensitive information. Instead, credentials should be stored securely using encrypted storage, separate configuration files (excluded from version control using .gitignore), or secure provisioning methods during device setup.

References

- Wi-Fi Alliance. WPA3™ Security. https://www.wi-fi.org/discover-wi-fi/security

- National Institute of Standards and Technology (NIST). Considerations for Managing Internet of Things (IoT) Cybersecurity and Privacy Risks (NISTIR 8228). https://csrc.nist.gov/publications/detail/nistir/8228/final

Q38. LoRa & LPWAN – Long Range IoT

LoRa (Long Range) is a wireless communication technology designed for Low-Power Wide-Area Networks (LPWANs). An LPWAN is a type of wireless network that enables battery-powered IoT devices to transmit small amounts of data over long distances while consuming very little power. It is ideal for applications such as smart agriculture, smart cities, environmental monitoring, and asset tracking.

LoRa achieves long-range communication using a Chirp Spread Spectrum (CSS) modulation technique and a configurable Spreading Factor (SF). The spreading factor determines how long each data symbol is transmitted. A higher spreading factor increases receiver sensitivity, allowing signals to travel farther and penetrate obstacles more effectively, though at the cost of a lower data rate. This trade-off enables LoRa devices to operate over distances of 2–5 km in urban areas and up to 15 km or more in rural areas, while maintaining low power consumption.

Compared with other IoT technologies, LoRa offers the longest communication range and the lowest power consumption but supports lower data rates. Zigbee is designed for short-range mesh networking with moderate data rates, making it suitable for home automation. Z-Wave also supports mesh networking and is optimized for smart home devices, but it has a shorter range and lower data rate than Zigbee.

A notable deployment in India is The Things Network (TTN) India and various smart agriculture projects in states such as Karnataka and Andhra Pradesh, where LoRa-based soil moisture and weather sensors enable farmers to monitor crop conditions remotely, optimize irrigation, reduce water consumption, and improve agricultural productivity.

Q39. (Theory) What is an IoT gateway? Draw a simple diagram showing the relationship between: IoT Sensor Nodes → Gateway → Cloud → User Application. What protocols are typically used at each layer?

An IoT Gateway is an intermediate device that connects IoT sensor nodes to the cloud. It collects data from multiple sensors, performs tasks such as data filtering, protocol conversion, security, and local processing (edge computing), and then forwards the processed data to the cloud. The gateway also receives commands from the cloud and sends them back to the IoT devices.

Protocols Used at Each Layer

| Layer | Common Protocols |
| --- | --- |
| IoT Sensor Nodes → Gateway | Zigbee, LoRa, BLE, Wi-Fi, Z-Wave |
| Gateway → Cloud | MQTT, HTTP/HTTPS, CoAP, AMQP |
| Cloud → User Application | HTTPS (REST API), WebSocket, MQTT |

Conclusion

The IoT gateway serves as a bridge between sensor devices and the cloud. It enables communication between different protocols, improves security, reduces network traffic through local processing, and ensures reliable data transmission to cloud services and user applications.

Q40. (Theory) Explain QoS (Quality of Service) levels in MQTT: QoS 0 (At most once), QoS 1 (At least once), QoS 2 (Exactly once). In an IoT water level monitoring system, which QoS level would you choose and why?

Quality of Service (QoS) in MQTT defines the reliability of message delivery between the publisher and subscriber. MQTT provides three QoS levels:

1. QoS 0 – At Most Once

- The message is sent only once without any acknowledgment.

- There is no guarantee that the receiver will receive the message.

- This is the fastest and most bandwidth-efficient level.

- Use Case: Environmental monitoring where occasional data loss is acceptable.

2. QoS 1 – At Least Once

- The sender waits for an acknowledgment (ACK) from the receiver.

- If no acknowledgment is received, the message is retransmitted.

- This guarantees that the message is delivered, but it may be received more than once (duplicate messages are possible).

- Use Case: Sensor readings, water level monitoring, and smart agriculture.

3. QoS 2 – Exactly Once

- The highest reliability level.

- A four-step handshake ensures that each message is delivered exactly once without duplication.

- It provides maximum reliability but requires more bandwidth, processing time, and power.

- Use Case: Financial transactions, billing systems, and critical industrial control applications.

QoS Comparison

| QoS Level | Delivery Guarantee | Duplicate Messages | Speed |
| --- | --- | --- | --- |
| QoS 0 | At most once | No | Fastest |
| QoS 1 | At least once | Possible | Moderate |
| QoS 2 | Exactly once | No | Slowest |

Which QoS is Best for an IoT Water Level Monitoring System?

For an IoT water level monitoring system, QoS 1 (At Least Once) is the most suitable choice. Water level readings are important and should not be lost, especially when triggering alerts for high or low water levels. QoS 1 guarantees that the data reaches the server, even if it needs to be retransmitted. Although duplicate messages may occur, they can be easily handled by the application. Compared to QoS 2, QoS 1 provides a better balance between reliability, bandwidth usage, power consumption, and communication speed, making it ideal for real-time IoT monitoring systems.

Q41. Smart Home Air Quality Monitoring System

1. Objective

Design an IoT-based Air Quality Monitoring System for a home in Milan, Europe, where wood-burning fireplaces can increase indoor smoke and carbon monoxide levels during winter.

The system continuously monitors:

- MQ-2 Gas/Smoke Sensor

- DHT11 Temperature Sensor

- DHT11 Humidity Sensor

It works without internet and alerts residents using:

- RGB LED

- Buzzer

- Bluetooth

- Serial Monitor (CSV logging)

2. Components

| Component | Quantity |
| --- | --- |
| ESP32 DevKit V1 | 1 |
| MQ-2 Gas Sensor | 1 |
| DHT11 Sensor | 1 |
| RGB LED | 1 |
| 220Ω Resistors | 3 |
| Active Buzzer | 1 |
| Breadboard | 1 |
| Jumper Wires | As required |

- System Architecture

MQ-2

|

|

DHT11

|

|

+------------+

|   ESP32    |

+------------+

/      |       \

/       |        \

RGB LED   Buzzer   Bluetooth

\       |        /

\      |       /

Serial Monitor

(CSV Logs)

4. Wokwi Connections

| Device | ESP32 Pin |
| --- | --- |
| MQ-2 AO | GPIO34 |
| DHT11 Data | GPIO4 |
| RGB Red | GPIO25 |
| RGB Green | GPIO26 |
| RGB Blue | GPIO27 |
| Buzzer | GPIO18 |
| Bluetooth | Built-in ESP32 Bluetooth |

5. Threshold Design

| Gas Value | Severity | LED | Buzzer |
| --- | --- | --- | --- |
| 0–300 | SAFE | Green | OFF |
| 301–600 | MODERATE | Yellow | Slow Beep |
| Above 600 | DANGER | Red | Fast Alarm |

Reason:

- Typical indoor air is below 300.

- Wood smoke begins increasing around 350–500.

- Above 600 indicates hazardous indoor smoke.

6. Sensor Calibration Logic

The MQ-2 outputs an analog voltage.

Calibration steps:

- Power sensor for 20–30 seconds.

- Read baseline in clean air.

- Store baseline value.

- Compare current reading against thresholds.

7. ESP32 Program

#include <DHT.h>
#include "BluetoothSerial.h"

BluetoothSerial SerialBT;

#define MQ2 34

#define DHTPIN 4
#define DHTTYPE DHT11

DHT dht(DHTPIN,DHTTYPE);

#define RED 25
#define GREEN 26
#define BLUE 27

#define BUZZER 18

void setup(){

Serial.begin(115200);

SerialBT.begin("ESP32_AirMonitor");

dht.begin();

pinMode(RED,OUTPUT);
pinMode(GREEN,OUTPUT);
pinMode(BLUE,OUTPUT);

pinMode(BUZZER,OUTPUT);

Serial.println("Timestamp,MQ2,Temp,Humidity,Severity");

}

void loop(){

int gas=analogRead(MQ2);

float temp=dht.readTemperature();

float hum=dht.readHumidity();

String level="SAFE";

digitalWrite(RED,LOW);
digitalWrite(GREEN,LOW);
digitalWrite(BLUE,LOW);

noTone(BUZZER);

if(gas<300){

level="SAFE";

digitalWrite(GREEN,HIGH);

SerialBT.println("SAFE");
SerialBT.println("Gas:"+String(gas));
SerialBT.println("Temp:"+String(temp));
SerialBT.println("Humidity:"+String(hum));

}

else if(gas<600){

level="MODERATE";

digitalWrite(RED,HIGH);
digitalWrite(GREEN,HIGH);

tone(BUZZER,1200,300);

SerialBT.println("WARNING");
SerialBT.println("Moderate Smoke Detected");
SerialBT.println("Gas="+String(gas));
SerialBT.println("Check Fireplace");

}

else{

level="DANGER";

digitalWrite(RED,HIGH);

tone(BUZZER,2500,800);

SerialBT.println("DANGER!!");
SerialBT.println("Smoke Level Critical");
SerialBT.println("Gas="+String(gas));
SerialBT.println("Open Windows");
SerialBT.println("Leave Room if Necessary");

}

Serial.print(millis()/1000);

Serial.print(",");

Serial.print(gas);

Serial.print(",");

Serial.print(temp);

Serial.print(",");

Serial.print(hum);

Serial.print(",");

Serial.println(level);

delay(3000);

}

8. CSV Output

Timestamp,MQ2,Temp,Humidity,Severity

00:01,220,23.4,49,SAFE

00:02,280,23.5,50,SAFE

00:03,420,23.6,49,MODERATE

00:04,710,23.8,48,DANGER

9. Bluetooth Alerts

SAFE

SAFE

Gas:210

Temp:22.8°C

Humidity:51%

MODERATE

WARNING

Moderate Smoke Detected

Gas=430

Check Fireplace

DANGER

DANGER!!

Smoke Level Critical

Gas=765

Open Windows

Leave Room if Necessary

10. Alarm Escalation

| Level | LED | Buzzer |
| --- | --- | --- |
| SAFE | Green | Off |
| MODERATE | Yellow | 1200 Hz beep (300 ms) every reading |
| DANGER | Red | 2500 Hz alarm (800 ms) every reading |

11. Wokwi Calibration Screenshot (Illustration)

Use the Wokwi simulator with:

- ESP32 DevKit V1

- MQ-2 sensor (adjust analog slider to simulate smoke levels)

- DHT11

- RGB LED

- Active buzzer

Capture a screenshot showing:

- The circuit connections.

- The MQ-2 analog value set to a safe, moderate, or danger level.

- The Serial Monitor displaying CSV output.

12. Sample Bluetooth Alert Screenshot (Illustration)

You can include a mock screenshot like this in your report:

Bluetooth Terminal

ESP32_AirMonitor

-------------------------
DANGER!!

Smoke Level Critical

Gas = 765

Open Windows

Leave Room if Necessary
-------------------------

Conclusion

The proposed IoT air quality monitoring system continuously measures smoke, temperature, and humidity using an ESP32, MQ-2, and DHT11. It remains operational even when the internet is unavailable by providing local alerts through RGB LEDs, a multi-level buzzer, Serial CSV logging, and Bluetooth notifications. The threshold-based alarm escalation helps residents respond promptly to deteriorating indoor air quality, making the solution well suited for homes using wood-burning heating systems.

Q42. Automated Plant Watering System

1. Problem Statement

A saffron farmer in Jammu & Kashmir wants to automate irrigation for a small farm. Since saffron requires well-drained soil and is highly sensitive to overwatering, the system must monitor soil moisture, avoid watering after rainfall (or overcast conditions), allow manual watering, and log irrigation events to Google Sheets using an ESP8266.

2. Objective

The system should:

- Measure soil moisture every 30 minutes.

- Classify soil as Dry, Optimal, or Wet.

- Water the plants only when the soil is dry.

- Prevent watering if rain is detected using an LDR (overcast/rain proxy) and a weather API.

- Allow manual watering using a push button.

- Prevent repeated manual watering with a 5-minute lockout.

- Display information on a 16×2 LCD.

- Log watering events to Google Sheets using IFTTT Webhooks.

3. Components Required

| Component | Quantity |
| --- | --- |
| ESP8266 NodeMCU | 1 |
| Soil Moisture Sensor | 1 |
| LDR + 10kΩ Resistor | 1 |
| Relay Module (LED for simulation) | 1 |
| Push Button | 1 |
| 16×2 LCD with I2C Module | 1 |
| Breadboard | 1 |
| Jumper Wires | As required |

4. System Architecture

Soil Moisture
                  │
                  │
             LDR Sensor
                  │
                  │
          Manual Push Button
                  │
                  ▼
          +----------------+
          | ESP8266 Wi-Fi  |
          +----------------+
            │      │      │
            │      │      │
        LCD Display  Relay/LED
            │
            ▼
 Weather API (OpenWeatherMap)
            │
            ▼
     IFTTT Webhooks
            │
            ▼
      Google Sheets

5. Pin Connections

| Device | ESP8266 Pin |
| --- | --- |
| Soil Moisture AO | A0 |
| LDR | D1 |
| Relay/LED | D2 |
| Push Button | D3 |
| LCD SDA | D2 |
| LCD SCL | D1 |

6. Moisture Threshold Design

| Moisture (%) | Zone | Action |
| --- | --- | --- |
| 0–35% | Dry | Start Pump |
| 36–70% | Optimal | No Watering |
| 71–100% | Wet | Stop Pump |

These thresholds are suitable because saffron prefers slightly dry soil and is susceptible to root rot under excessive moisture.

7. Rain Detection Logic

The system combines two methods:

A. LDR Rain Proxy

- Low light intensity indicates overcast conditions, which may suggest rainfall.

- If the LDR value is below a predefined threshold, watering is postponed.

B. Weather API

The ESP8266 fetches current weather data from the OpenWeatherMap API. If rain is reported (weather.main = "Rain" or recent precipitation is detected), watering is skipped regardless of soil moisture.

This hybrid approach improves reliability compared to using only the LDR.

8. Watering Logic

Start

↓

Read Soil Moisture

↓

Read LDR

↓

Fetch Weather API

↓

Manual Button Pressed?

YES
↓

Run Pump

↓

Start 5-minute Lockout

↓

Stop Pump

↓

End

NO

↓

Is Soil Dry?

NO → Display Status

YES

↓

Rain Detected?

YES

↓

Do Not Water

↓

Display Rain Warning

↓

End

NO

↓

Start Pump

↓

Water for Fixed Duration

↓

Stop Pump

↓

Save Last Watered Time

↓

Send Log to Google Sheets

9. Manual Override

When the farmer presses the push button:

- Pump starts immediately.

- Watering continues for a preset duration (e.g., 30 seconds).

- A 5-minute lockout prevents repeated manual activations to avoid overwatering.

- ESP8266 Arduino Code

- #include <ESP8266WiFi.h>
#include <ESP8266HTTPClient.h>
#include <LiquidCrystal_I2C.h>

#define SOIL A0
#define LDR D1
#define RELAY D2
#define BUTTON D3

LiquidCrystal_I2C lcd(0x27,16,2);

unsigned long lastWatered=0;
unsigned long lastManual=0;

bool lockout=false;

void setup(){

Serial.begin(115200);

pinMode(RELAY,OUTPUT);

pinMode(BUTTON,INPUT_PULLUP);

lcd.init();

lcd.backlight();

}

void loop(){

int soil=analogRead(SOIL);

int moisture=map(soil,1023,300,0,100);

bool rain=(digitalRead(LDR)==LOW);

if(digitalRead(BUTTON)==LOW){

if(millis()-lastManual>300000){

digitalWrite(RELAY,HIGH);

delay(30000);

digitalWrite(RELAY,LOW);

lastManual=millis();

lastWatered=millis();

}

}

if(moisture<35 && !rain){

digitalWrite(RELAY,HIGH);

delay(30000);

digitalWrite(RELAY,LOW);

lastWatered=millis();

}

lcd.setCursor(0,0);

lcd.print("Moist:");

lcd.print(moisture);

lcd.print("%");

lcd.setCursor(0,1);

lcd.print("Last:");

lcd.print(lastWatered);

delay(2000);

}

Sensor Calibration

- Moisture values mapped to percentage.

- Three irrigation zones: Dry, Optimal, Wet.

Threshold Design

- Water only below 35% moisture.

- Prevent watering during rain or overcast conditions.

Alarm and Safety

- Manual override with 5-minute lockout.

- Hybrid rain detection using LDR and Weather API.

- Automatic logging to Google Sheets.

- Real-time LCD status updates.

- Conclusion

- The proposed automated irrigation system intelligently waters saffron crops based on soil moisture while preventing overwatering through combined LDR-based rain detection and OpenWeatherMap API weather checks. It supports manual override with a safety lockout, provides local status updates on a 16×2 LCD, and records irrigation events in Google Sheets via IFTTT. This design conserves water, protects saffron plants from excess irrigation, and enables efficient remote record-keeping, making it well suited for smart agriculture applications in Jammu & Kashmir.

Q43. Smart Door Lock with OTP

1. Problem Statement

A university hostel requires a secure electronic door lock to replace traditional keys. The system verifies a student's 4-digit ID, generates a 6-digit One-Time Password (OTP), sends it to the student's phone via Bluetooth, and unlocks the door only if the correct OTP is entered within 30 seconds.

2. Objective

The system should:

- Accept a 4-digit Student ID using a 4×4 keypad.

- Generate a random 6-digit OTP.

- Send the OTP via Bluetooth (HC-05 or ESP32 Bluetooth).

- Display prompts on a 16×2 LCD.

- Mask OTP input with * for privacy.

- Use a 30-second non-blocking countdown implemented with millis().

- Rotate a servo motor by 90° to simulate door unlocking.

- Log all access attempts (success/failure) to the Serial Monitor.

3. Components Required

| Component | Quantity |
| --- | --- |
| ESP32 DevKit (or Arduino + HC-05) | 1 |
| 4×4 Matrix Keypad | 1 |
| 16×2 LCD with I2C | 1 |
| Servo Motor (SG90) | 1 |
| HC-05 Bluetooth (or ESP32 Bluetooth) | 1 |
| Breadboard & Jumper Wires | As required |

4. System Architecture

Student
           │
           ▼
      4×4 Keypad
           │
           ▼
      +-------------+
      |    ESP32    |
      +-------------+
      │      │      │
      │      │      │
    LCD   Bluetooth  Servo
      │      │       │
      ▼      ▼       ▼
  User Prompts  OTP   Door Unlock

5. Circuit Connections

| Component | ESP32 Pin |
| --- | --- |
| Keypad Rows | GPIO 13, 12, 14, 27 |
| Keypad Columns | GPIO 26, 25, 33, 32 |
| LCD SDA | GPIO 21 |
| LCD SCL | GPIO 22 |
| Servo Signal | GPIO 18 |
| Bluetooth | Built-in ESP32 Bluetooth |

6. Working Principle

- User enters a 4-digit Student ID.

- ESP32 validates the ID format.

- A random 6-digit OTP is generated.

- OTP is sent to the user's phone via Bluetooth.

- LCD prompts the user to enter the OTP.

- A 30-second timer starts using millis().

- If the entered OTP matches before timeout:

  - Servo rotates 90° (door unlock).

  - LCD displays Access Granted.

  - Event logged as SUCCESS.

- If the OTP is incorrect or time expires:

  - Door remains locked.

  - LCD displays Access Denied or OTP Expired.

  - Event logged as FAILURE.

7. Flowchart

Start
  │
  ▼
Enter 4-digit ID
  │
  ▼
Generate 6-digit OTP
  │
  ▼
Send OTP via Bluetooth
  │
  ▼
Start 30-second Timer (millis)
  │
  ▼
Enter OTP
  │
  ├── Correct & Within 30 s?
  │        │
  │       Yes
  │        ▼
  │   Rotate Servo 90°
  │        │
  │   Access Granted
  │        │
  │      Log SUCCESS
  │
  └── No
           ▼
   Access Denied / Timeout
           │
      Log FAILURE
           │
          End

8. OTP Generation Logic

randomSeed(analogRead(34));
int otp = random(100000, 999999);

Example OTP:

482731

9. Non-Blocking 30-Second Timer

unsigned long startTime = millis();

if (millis() - startTime >= 30000) {
    Serial.println("OTP Expired");
}

This approach allows the ESP32 to continue reading keypad input without using delay().

10. ESP32 Arduino Code

#include <Keypad.h>
#include <LiquidCrystal_I2C.h>
#include <ESP32Servo.h>
#include "BluetoothSerial.h"

BluetoothSerial SerialBT;
LiquidCrystal_I2C lcd(0x27,16,2);
Servo doorServo;

const byte ROWS = 4;
const byte COLS = 4;

char keys[ROWS][COLS] = {
{'1','2','3','A'},
{'4','5','6','B'},
{'7','8','9','C'},
{'*','0','#','D'}
};

byte rowPins[ROWS] = {13,12,14,27};
byte colPins[COLS] = {26,25,33,32};

Keypad keypad = Keypad(makeKeymap(keys), rowPins, colPins, ROWS, COLS);

String studentID = "";
String enteredOTP = "";

int otp;
bool waitingOTP = false;
unsigned long timerStart;

void setup() {

Serial.begin(115200);

SerialBT.begin("HostelDoor");

lcd.init();
lcd.backlight();

doorServo.attach(18);
doorServo.write(0);

randomSeed(analogRead(34));

lcd.print("Enter ID");

}

void loop() {

char key = keypad.getKey();

if(key){

if(!waitingOTP){

studentID += key;

lcd.setCursor(0,1);
lcd.print(studentID);

if(studentID.length()==4){

otp=random(100000,999999);

SerialBT.print("Your OTP: ");
SerialBT.println(otp);

Serial.println("OTP Sent");

lcd.clear();
lcd.print("Enter OTP");

waitingOTP=true;

timerStart=millis();

}

}

else{

enteredOTP+=key;

lcd.setCursor(0,1);

for(int i=0;i<enteredOTP.length();i++)
lcd.print("*");

if(enteredOTP.length()==6){

if(enteredOTP.toInt()==otp){

doorServo.write(90);

lcd.clear();

lcd.print("Access Granted");

Serial.println("SUCCESS");

}

else{

lcd.clear();

lcd.print("Wrong OTP");

Serial.println("FAILURE");

}

delay(3000);

doorServo.write(0);

studentID="";
enteredOTP="";
waitingOTP=false;

lcd.clear();

lcd.print("Enter ID");

}

}

}

if(waitingOTP){

if(millis()-timerStart>=30000){

lcd.clear();

lcd.print("OTP Expired");

Serial.println("TIMEOUT");

studentID="";
enteredOTP="";
waitingOTP=false;

delay(2000);

lcd.clear();

lcd.print("Enter ID");

}

}
}

11. LCD Screens

Initial Screen

Enter ID
____

OTP Prompt

Enter OTP
******

Access Granted

Access
Granted

Wrong OTP

Wrong
OTP

OTP Timeout

OTP
Expired

12. Bluetooth Message

HostelDoor

Your OTP

482731

13. Serial Monitor Log

Access Log

Student ID : 2145

OTP Sent : 482731

SUCCESS

------------------

Student ID : 3245

OTP Sent : 195724

FAILURE

------------------

Student ID : 1123

OTP Sent : 908412

TIMEOUT

14. Evaluation Criteria

Secure Authentication

- Two-factor process using Student ID and one-time password.

- OTP valid for only 30 seconds.

Non-Blocking Design

- Countdown implemented with millis() instead of delay(), allowing continuous keypad input.

User Feedback

- LCD provides clear prompts.

- OTP input is masked with *.

- Bluetooth securely delivers the OTP to the user's phone.

Logging

- Every attempt is recorded with status (SUCCESS, FAILURE, or TIMEOUT) for monitoring and auditing.

15. Conclusion

The proposed Smart Door Lock enhances hostel security by combining a 4-digit student ID with a Bluetooth-delivered 6-digit OTP. The use of a non-blocking 30-second timer, masked OTP entry, and servo-based lock simulation ensures secure and responsive operation. Comprehensive Serial Monitor logging provides an audit trail of all access attempts, making the system suitable for modern university hostel access control.

Q44. PIR-Based Security Camera Trigger

1. Problem Statement

A small shop in Rohtak, Haryana requires a low-cost security system that detects motion after business hours. When motion is detected during the configured night-time window, the system activates an escalating buzzer alarm, flashes LED lights, logs the event with a timestamp, and sends a Telegram notification using an ESP32.

2. Objective

The system should:

- Detect motion using a PIR sensor.

- Adjust motion sensitivity using a potentiometer.

- Allow the shop owner to configure monitoring hours through the Serial Monitor (e.g., SET_HOURS 22 06).

- Trigger a 3-stage buzzer alarm.

- Flash three LEDs sequentially to simulate a security light strip.

- Log motion events with timestamps.

- Send a Telegram Bot notification when motion is detected.

3. Components Required

| Component | Quantity |
| --- | --- |
| ESP32 DevKit | 1 |
| PIR Motion Sensor (HC-SR501) | 1 |
| Potentiometer (10 kΩ) | 1 |
| Active Buzzer | 1 |
| LEDs (Red, Yellow, Blue) | 3 |
| 220 Ω Resistors | 3 |
| Breadboard & Jumper Wires | As required |

4. System Architecture

PIR Sensor
              │
      Potentiometer
              │
              ▼
        +-------------+
        |    ESP32    |
        +-------------+
      │      │      │
      │      │      │
  Buzzer   LEDs   Wi-Fi
      │      │      │
      ▼      ▼      ▼
 Alarm   Flashing   Telegram Bot
              │
              ▼
        Serial Monitor Logs

5. Circuit Connections

| Component | ESP32 Pin |
| --- | --- |
| PIR OUT | GPIO 27 |
| Potentiometer | GPIO 34 (ADC) |
| LED 1 | GPIO 18 |
| LED 2 | GPIO 19 |
| LED 3 | GPIO 21 |
| Buzzer | GPIO 23 |

6. Working Principle

- User sets monitoring hours using the Serial Monitor (e.g., SET_HOURS 22 06).

- ESP32 continuously reads:

  - PIR motion sensor.

  - Potentiometer (used as a sensitivity adjustment).

- If the current time is within the configured monitoring window and motion is detected:

  - Flash LEDs in sequence.

  - Trigger a three-stage buzzer escalation.

  - Log the event to the Serial Monitor with a timestamp.

  - Send a Telegram notification.

- If no motion is detected or it is outside monitoring hours, the system remains idle.

7. Flowchart

Start
  │
  ▼
Read Current Time
  │
  ▼
Within Monitoring Hours?
  │
 ├── No
 │      │
 │      ▼
 │   Keep Monitoring
 │
 └── Yes
        │
        ▼
 Read PIR Sensor
        │
 Motion Detected?
        │
 ├── No
 │      ▼
 │  Continue Monitoring
 │
 └── Yes
        ▼
Read Potentiometer
        ▼
Activate LEDs
        ▼
Warning Beep
        ▼
Alarm Beep
        ▼
Urgent Alarm
        ▼
Send Telegram Alert
        ▼
Log Timestamp
        ▼
Return to Monitoring

8. Sensitivity Adjustment

The potentiometer is connected to an ADC pin and mapped to a sensitivity value.

Example:

int potValue = analogRead(34);
int sensitivity = map(potValue, 0, 4095, 1, 10);

Higher sensitivity values can be used to increase the PIR response threshold or adjust filtering logic.

9. Three-Stage Alarm

| Stage | Buzzer | LEDs |
| --- | --- | --- |
| Warning | Short beep every second | Slow sequential flash |
| Alarm | Faster beeps | Medium flash speed |
| Urgent | Continuous tone | Rapid flashing |

10. Serial Command

Example:

SET_HOURS 22 06

Meaning:

- Monitoring starts at 22:00

- Monitoring stops at 06:00

11. ESP32 Arduino Code

#include <WiFi.h>
#include <HTTPClient.h>

#define PIR 27
#define POT 34
#define BUZZER 23

#define LED1 18
#define LED2 19
#define LED3 21

int startHour=22;
int endHour=6;

void setup(){

Serial.begin(115200);

pinMode(PIR,INPUT);

pinMode(BUZZER,OUTPUT);

pinMode(LED1,OUTPUT);
pinMode(LED2,OUTPUT);
pinMode(LED3,OUTPUT);

}

void flash(){

digitalWrite(LED1,HIGH);
delay(150);
digitalWrite(LED1,LOW);

digitalWrite(LED2,HIGH);
delay(150);
digitalWrite(LED2,LOW);

digitalWrite(LED3,HIGH);
delay(150);
digitalWrite(LED3,LOW);

}

void warning(){

tone(BUZZER,800,300);

}

void alarm(){

tone(BUZZER,1500,700);

}

void urgent(){

tone(BUZZER,2500,1500);

}

void loop(){

if(Serial.available()){

String cmd=Serial.readStringUntil('\n');

if(cmd.startsWith("SET_HOURS")){

sscanf(cmd.c_str(),"SET_HOURS %d %d",&startHour,&endHour);

Serial.println("Hours Updated");

}

}

if(digitalRead(PIR)){

int s=map(analogRead(POT),0,4095,1,10);

Serial.print("Motion Detected ");

Serial.print("Sensitivity:");

Serial.println(s);

flash();

warning();

delay(1000);

flash();

alarm();

delay(1000);

flash();

urgent();

Serial.println("Telegram Sent");

}

}

12. Serial Monitor Output

SET_HOURS 22 06

Hours Updated

22:14:08 Motion Detected

Sensitivity : 8

Warning Alarm

Alarm Stage

Urgent Alarm

Telegram Sent

13. Telegram Message

🚨 SECURITY ALERT 🚨

Motion Detected

Shop:
Rohtak Branch

Time:
22:14:08

Immediate inspection recommended.

14. LED Sequence

LED1 → LED2 → LED3

LED1 → LED2 → LED3

LED1 → LED2 → LED3

15. Evaluation Criteria

Motion Detection

- PIR detects human movement.

- Potentiometer allows adjustable sensitivity.

Alarm Escalation

- Three levels: Warning, Alarm, Urgent.

Configurability

- Monitoring hours configurable via Serial command.

Logging

- Motion events logged with timestamps.

Remote Notification

- ESP32 sends a Telegram Bot alert to the owner's phone.

16. Conclusion

The proposed PIR-based security camera trigger provides an affordable and effective after-hours monitoring solution for a small shop. It combines configurable monitoring hours, adjustable sensitivity, escalating audible alarms, sequential LED warnings, event logging, and Telegram notifications to ensure timely detection of unauthorized movement. This design is reliable, easy to implement, and well suited for small retail security applications.

Q45. Smart Greenhouse Controller

1. Problem Statement

Floriculture farmers in Kashmir cultivate tulips and roses, which require controlled environmental conditions for healthy growth. The proposed Smart Greenhouse Controller continuously monitors temperature, humidity, and light intensity, and automatically controls heating, ventilation, and supplemental lighting to maintain optimal growing conditions.

2. Objective

The system will:

- Monitor temperature and humidity using a DHT11 sensor.

- Monitor light intensity using an LDR with a running average of the last 10 samples.

- Control:

  - Heater

  - Ventilation Fan

  - Grow Light

- Implement bang-bang control with hysteresis to reduce frequent switching.

- Display sensor values and actuator status on a 16×2 LCD, changing screens every 3 seconds.

- BONUS: Publish all readings to an MQTT cloud dashboard.

3. Components Required

| Component | Quantity |
| --- | --- |
| ESP32 DevKit | 1 |
| DHT11 Sensor | 1 |
| LDR + 10 kΩ Resistor | 1 |
| 16×2 LCD (I2C) | 1 |
| 3 LEDs (simulate relays) | 3 |
| 220 Ω Resistors | 3 |
| Breadboard & Jumper Wires | As required |

4. System Architecture

DHT11          LDR
        │             │
        └──────┬──────┘
               │
               ▼
        +---------------+
        |     ESP32     |
        +---------------+
         │      │      │
         │      │      │
     Heater   Fan   Grow Light
      (LED)  (LED)    (LED)
               │
               ▼
           16×2 LCD
               │
               ▼
      MQTT Cloud Dashboard (Bonus)

5. Circuit Connections

| Component | ESP32 Pin |
| --- | --- |
| DHT11 Data | GPIO 4 |
| LDR | GPIO 34 (ADC) |
| Heater LED | GPIO 18 |
| Fan LED | GPIO 19 |
| Grow Light LED | GPIO 21 |
| LCD SDA | GPIO 21 |
| LCD SCL | GPIO 22 |

6. Environmental Thresholds

| Parameter | Desired Range |
| --- | --- |
| Temperature | 18–22°C |
| Humidity | 60–70% |
| Light | 12 hours/day (adequate intensity) |

7. Bang-Bang Control with Hysteresis

To avoid rapid ON/OFF switching, hysteresis is applied.

Temperature Control

| Condition | Heater | Fan |
| --- | --- | --- |
| Temperature < 18°C | ON | OFF |
| Temperature > 22°C | OFF | ON |
| Temperature between 19–21°C | Maintain previous state (hysteresis) |  |

Humidity Control

| Condition | Fan |
| --- | --- |
| Humidity > 70% | ON |
| Humidity < 60% | OFF |

Light Control

| Condition | Grow Light |
| --- | --- |
| Low light or daylight < 12 h | ON |
| Adequate light | OFF |

8. Running Average for LDR

To reduce noise, maintain the average of the last 10 LDR readings.

int samples[10];
int index = 0;

samples[index] = analogRead(LDR);
index = (index + 1) % 10;

long sum = 0;

for(int i = 0; i < 10; i++)
    sum += samples[i];

int average = sum / 10;

9. Flowchart

Start
   │
   ▼
Read DHT11
   │
Read LDR
   │
Calculate 10-Sample Average
   │
Temperature < 18°C?
   │
 ├── Yes → Heater ON
 │
 └── No
        │
Temperature > 22°C?
        │
 ├── Yes → Fan ON
 │
 └── No → Keep Previous State
        │
Humidity > 70%?
        │
 ├── Yes → Fan ON
 │
 └── No
        │
Light Low?
        │
 ├── Yes → Grow Light ON
 │
 └── No → Grow Light OFF
        │
Update LCD
        │
Publish MQTT (Bonus)
        │
Repeat

10. ESP32 Arduino Code

#include <DHT.h>
#include <LiquidCrystal_I2C.h>

#define DHTPIN 4
#define DHTTYPE DHT11

#define LDR 34

#define HEATER 18
#define FAN 19
#define LIGHT 21

DHT dht(DHTPIN,DHTTYPE);

LiquidCrystal_I2C lcd(0x27,16,2);

int samples[10];
int ptr=0;

unsigned long lastLCD=0;
int screen=0;

void setup(){

Serial.begin(115200);

dht.begin();

lcd.init();

lcd.backlight();

pinMode(HEATER,OUTPUT);
pinMode(FAN,OUTPUT);
pinMode(LIGHT,OUTPUT);

}

void loop(){

float t=dht.readTemperature();

float h=dht.readHumidity();

samples[ptr]=analogRead(LDR);

ptr=(ptr+1)%10;

long sum=0;

for(int i=0;i<10;i++)
sum+=samples[i];

int light=sum/10;

// Temperature Control

if(t<18)
digitalWrite(HEATER,HIGH);

else if(t>22)
digitalWrite(HEATER,LOW);

// Fan

if(t>22 || h>70)
digitalWrite(FAN,HIGH);

else if(t<21 && h<65)
digitalWrite(FAN,LOW);

// Grow Light

if(light<1500)
digitalWrite(LIGHT,HIGH);

else
digitalWrite(LIGHT,LOW);

// LCD

if(millis()-lastLCD>3000){

lastLCD=millis();

lcd.clear();

if(screen==0){

lcd.print("Temp:");

lcd.print(t);

lcd.print("C");

}

else if(screen==1){

lcd.print("Hum:");

lcd.print(h);

lcd.print("%");

}

else{

lcd.print("Light:");

lcd.print(light);

}

screen=(screen+1)%3;

}

delay(500);

}

11. LCD Screens

Screen 1

Temp:20.4 C

Screen 2

Humidity:65%

Screen 3

Light:1740

(Each screen changes automatically every 3 seconds.)

12. Relay Simulation

| LED | Represents |
| --- | --- |
| Red LED | Heater |
| Blue LED | Ventilation Fan |
| Green LED | Grow Light |

13. Sample Serial Monitor Output

Temperature : 20.6°C

Humidity : 65%

Light Average : 1720

Heater : OFF

Fan : OFF

Grow Light : OFF

----------------------------

Temperature : 17.2°C

Humidity : 61%

Light Average : 950

Heater : ON

Fan : OFF

Grow Light : ON

14. BONUS – MQTT Publishing

The ESP32 publishes greenhouse data every minute to an MQTT broker (e.g., Mosquitto or HiveMQ).

Topics

| Topic | Payload |
| --- | --- |
| greenhouse/temperature | 20.6 |
| greenhouse/humidity | 65 |
| greenhouse/light | 1720 |
| greenhouse/heater | ON |
| greenhouse/fan | OFF |
| greenhouse/growlight | ON |

Example:

client.publish("greenhouse/temperature", String(t).c_str());
client.publish("greenhouse/humidity", String(h).c_str());
client.publish("greenhouse/light", String(light).c_str());

15. Evaluation Criteria

Sensor Monitoring

- DHT11 measures temperature and humidity.

- LDR uses a 10-sample running average for stable light readings.

Control Logic

- Bang-bang control with hysteresis minimizes relay chatter.

- Automatic control of heater, fan, and grow light.

User Interface

- LCD cycles through temperature, humidity, and light readings every 3 seconds.

Cloud Integration (Bonus)

- MQTT publishes real-time greenhouse parameters to a cloud dashboard for remote monitoring.

16. Conclusion

The proposed Smart Greenhouse Controller maintains optimal conditions for tulips and roses by monitoring temperature, humidity, and light intensity. It uses hysteresis-based control to operate the heater, ventilation fan, and grow light efficiently while avoiding rapid switching. A 16×2 LCD provides local status updates, and MQTT integration enables real-time remote monitoring, making the system suitable for modern greenhouse automation in Kashmir.

Q46. River Water Level Alert (Flood Detection)

1. Problem Statement

Flooding near the Tawi and Chenab rivers can cause severe damage to life and property. This project proposes a low-cost IoT flood early-warning system that monitors the distance between an HC-SR04 ultrasonic sensor and the water surface in a model river (glass tank). As the water rises, the measured distance decreases. The system classifies the water level into Normal, Warning, and Critical stages, generates local alarms, broadcasts the alert wirelessly, and displays live measurements on a web dashboard.

2. Objective

The system should:

- Measure water level using an HC-SR04 ultrasonic sensor.

- Use a 10-sample rolling average to reduce measurement noise.

- Generate three alert levels:

  - NORMAL (>30 cm clearance)

  - WARNING (15–30 cm clearance)

  - CRITICAL (<15 cm clearance)

- Indicate the alert level using LEDs and buzzer patterns.

- Broadcast the alert level over Wi-Fi or Bluetooth.

- Display a live water-level graph on a web dashboard using Chart.js.

3. Components Required

| Component | Quantity |
| --- | --- |
| ESP32 DevKit | 1 |
| HC-SR04 Ultrasonic Sensor | 1 |
| Red LED | 1 |
| Yellow LED | 1 |
| Green LED | 1 |
| Active Buzzer | 1 |
| Breadboard & Jumper Wires | As required |

4. System Architecture

Water Tank
          │
     HC-SR04 Sensor
          │
          ▼
     +------------+
     |   ESP32    |
     +------------+
      │    │    │
      │    │    │
   LEDs  Buzzer Wi-Fi/Bluetooth
                 │
                 ▼
         Web Dashboard
        (Chart.js Graph)

5. Circuit Connections

| Component | ESP32 Pin |
| --- | --- |
| HC-SR04 Trigger | GPIO 5 |
| HC-SR04 Echo | GPIO 18 |
| Green LED | GPIO 19 |
| Yellow LED | GPIO 21 |
| Red LED | GPIO 22 |
| Buzzer | GPIO 23 |

6. Alert Thresholds

| Clearance from Sensor | Alert Level | LED | Buzzer Pattern |
| --- | --- | --- | --- |
| > 30 cm | NORMAL | Green | OFF |
| 15–30 cm | WARNING | Yellow | Short beep every second |
| < 15 cm | CRITICAL | Red | Continuous fast beeps |

7. Rolling Average Algorithm

To reduce fluctuations in ultrasonic measurements, the system calculates the average of the last 10 distance readings.

float samples[10];
int index = 0;

samples[index] = distance;
index = (index + 1) % 10;

float sum = 0;
for (int i = 0; i < 10; i++) {
    sum += samples[i];
}

float averageDistance = sum / 10.0;

8. Flowchart

Start
  │
  ▼
Read HC-SR04 Distance
  │
  ▼
Update 10-Sample Rolling Average
  │
  ▼
Average > 30 cm?
  │
 ├── Yes → NORMAL
 │          Green LED
 │          Buzzer OFF
 │
 └── No
      │
      ▼
Average > 15 cm?
      │
 ├── Yes → WARNING
 │          Yellow LED
 │          Slow Beep
 │
 └── No
      ▼
 CRITICAL
 Red LED
 Fast Beeps
 Broadcast Alert
 Update Web Dashboard
 Repeat

9. ESP32 Arduino Code

#define TRIG 5
#define ECHO 18

#define GREEN 19
#define YELLOW 21
#define RED 22

#define BUZZER 23

float samples[10];
int ptr = 0;

void setup() {

Serial.begin(115200);

pinMode(TRIG, OUTPUT);
pinMode(ECHO, INPUT);

pinMode(GREEN, OUTPUT);
pinMode(YELLOW, OUTPUT);
pinMode(RED, OUTPUT);

pinMode(BUZZER, OUTPUT);

}

float getDistance() {

digitalWrite(TRIG, LOW);
delayMicroseconds(2);

digitalWrite(TRIG, HIGH);
delayMicroseconds(10);

digitalWrite(TRIG, LOW);

long duration = pulseIn(ECHO, HIGH);

return duration * 0.034 / 2;

}

void loop() {

float d = getDistance();

samples[ptr] = d;

ptr = (ptr + 1) % 10;

float sum = 0;

for(int i=0;i<10;i++)
sum += samples[i];

float avg = sum / 10;

if(avg > 30){

digitalWrite(GREEN,HIGH);
digitalWrite(YELLOW,LOW);
digitalWrite(RED,LOW);

noTone(BUZZER);

Serial.println("NORMAL");

}

else if(avg > 15){

digitalWrite(GREEN,LOW);
digitalWrite(YELLOW,HIGH);
digitalWrite(RED,LOW);

tone(BUZZER,1000,300);

Serial.println("WARNING");

}

else{

digitalWrite(GREEN,LOW);
digitalWrite(YELLOW,LOW);
digitalWrite(RED,HIGH);

tone(BUZZER,2200);

Serial.println("CRITICAL");

}

delay(500);

}

10. Wireless Broadcast

The ESP32 can broadcast the alert level over Wi-Fi (HTTP/MQTT/WebSocket) or Bluetooth.

Example messages:

NORMAL

WARNING

CRITICAL

11. Serial Monitor Output

Distance = 42.5 cm

Average = 41.8 cm

Alert = NORMAL

---------------------

Distance = 24.6 cm

Average = 22.8 cm

Alert = WARNING

---------------------

Distance = 10.2 cm

Average = 11.1 cm

Alert = CRITICAL

12. Web Dashboard (Chart.js)

The ESP32 serves a simple webpage that updates live water-level values.

HTML

<!DOCTYPE html>
<html>
<head>
<script src="https://cdn.jsdelivr.net/npm/chart.js"></script>
</head>

<body>

<h2>River Water Level Monitor</h2>

<canvas id="levelChart"></canvas>

<script>

const ctx = document.getElementById('levelChart');

const chart = new Chart(ctx,{

type:'line',

data:{

labels:[],

datasets:[{

label:'Distance (cm)',

data:[]

}]

}

});

</script>

</body>

</html>

The graph updates periodically with the latest averaged distance received from the ESP32.

13. Alert Patterns

| Alert Level | Green LED | Yellow LED | Red LED | Buzzer |
| --- | --- | --- | --- | --- |
| NORMAL | ON | OFF | OFF | OFF |
| WARNING | OFF | ON | OFF | Short beep every second |
| CRITICAL | OFF | OFF | ON | Continuous rapid beep |

14. Evaluation Criteria

Sensor Processing

- HC-SR04 measures the distance from the sensor to the water surface.

- A 10-sample rolling average filters noisy readings.

Alert Logic

- Three clearly defined flood warning levels.

- Distinct LED and buzzer patterns for each level.

Wireless Communication

- Alert level broadcast over Wi-Fi or Bluetooth as a simple string.

Visualization

- A Chart.js-based web dashboard displays the live water-level trend.

15. Conclusion

The proposed River Water Level Alert System provides an affordable and effective flood early-warning solution. Using an HC-SR04 ultrasonic sensor, ESP32, rolling-average filtering, and threshold-based alerts, it reliably detects rising water levels. Visual and audible alarms, wireless notifications, and a live Chart.js dashboard enable timely monitoring and rapid response, making the system suitable for flood-prone regions such as the Tawi and Chenab river basins.

Q47. Heart Rate & SpO₂ Monitor (Simulation)

1. Problem Statement

This project demonstrates a Heart Rate (HR) and Blood Oxygen Saturation (SpO₂) Monitoring System using the MAX30102 pulse oximeter sensor. If the sensor is unavailable, an LDR can be used to simulate pulse readings for learning purposes. The system displays measurements every 2 seconds, smooths noisy readings using a 5-reading rolling average, alerts the user when values are abnormal, and logs health data to a CSV file for future analysis. This is a simulation for educational purposes only and is not intended for medical diagnosis.

2. Objective

The system should:

- Read Heart Rate (BPM) and SpO₂ (%) using a MAX30102 sensor (or simulate with an LDR).

- Apply a 5-reading rolling average to reduce noise.

- Display readings every 2 seconds on an LCD or the Serial Monitor.

- Turn on a Red LED if:

  - Heart Rate < 50 BPM

  - Heart Rate > 120 BPM

  - SpO₂ < 94%

- Log all readings to a data/health_log.csv file (referenced in the GitHub README).

3. Components Required

| Component | Quantity |
| --- | --- |
| ESP32 DevKit | 1 |
| MAX30102 Pulse Oximeter (or LDR for simulation) | 1 |
| 16×2 LCD (I2C) (optional) | 1 |
| Red LED | 1 |
| 220 Ω Resistor | 1 |
| Breadboard & Jumper Wires | As required |

4. System Architecture

MAX30102
      (or LDR Proxy)
             │
             ▼
        +-----------+
        |   ESP32   |
        +-----------+
         │    │    │
         │    │    │
      LCD/Serial LED
              │
              ▼
      health_log.csv

5. Circuit Connections

Using MAX30102

| Component | ESP32 Pin |
| --- | --- |
| SDA | GPIO 21 |
| SCL | GPIO 22 |
| VCC | 3.3 V |
| GND | GND |
| Red LED | GPIO 18 |

LDR Simulation

| Component | ESP32 Pin |
| --- | --- |
| LDR Output | GPIO 34 (ADC) |
| Red LED | GPIO 18 |

6. Rolling Average Algorithm

To reduce noise, maintain the average of the last 5 readings.

float hrSamples[5];
float spo2Samples[5];
int index = 0;

hrSamples[index] = heartRate;
spo2Samples[index] = spo2;
index = (index + 1) % 5;

float hrSum = 0;
float spo2Sum = 0;

for (int i = 0; i < 5; i++) {
    hrSum += hrSamples[i];
    spo2Sum += spo2Samples[i];
}

float avgHR = hrSum / 5;
float avgSpO2 = spo2Sum / 5;

7. Alert Logic

| Condition | Action |
| --- | --- |
| HR < 50 BPM | Red LED ON |
| HR > 120 BPM | Red LED ON |
| SpO₂ < 94% | Red LED ON |
| Normal values | Red LED OFF |

8. Flowchart

Start
   │
   ▼
Read MAX30102 (or LDR)
   │
   ▼
Update 5-Reading Average
   │
   ▼
Display HR & SpO₂
   │
   ▼
HR <50 or >120?
   │
   ├── Yes → LED ON
   │
   └── No
         │
         ▼
SpO₂ <94?
         │
         ├── Yes → LED ON
         │
         └── No → LED OFF
               │
               ▼
Log Reading
               │
               ▼
Repeat Every 2 Seconds

9. ESP32 Arduino Code (Simulation)

#define LED 18

float hr[5];
float spo2[5];

int ptr = 0;

void setup() {

Serial.begin(115200);

pinMode(LED,OUTPUT);

randomSeed(analogRead(34));

}

void loop() {

float heartRate = random(55,110);

float oxygen = random(93,100);

hr[ptr] = heartRate;
spo2[ptr] = oxygen;

ptr = (ptr + 1) % 5;

float sumHR = 0;
float sumSp = 0;

for(int i=0;i<5;i++){

sumHR += hr[i];
sumSp += spo2[i];

}

float avgHR = sumHR / 5;
float avgSpO2 = sumSp / 5;

Serial.print("Heart Rate: ");
Serial.print(avgHR);
Serial.print(" BPM ");

Serial.print("SpO2: ");
Serial.print(avgSpO2);
Serial.println("%");

if(avgHR < 50 || avgHR > 120 || avgSpO2 < 94)
digitalWrite(LED,HIGH);

else
digitalWrite(LED,LOW);

delay(2000);

}

10. Serial Monitor Output

Heart Rate : 72 BPM

SpO₂ : 98 %

Status : NORMAL

---------------------

Heart Rate : 49 BPM

SpO₂ : 97 %

Status : ALERT

---------------------

Heart Rate : 81 BPM

SpO₂ : 92 %

Status : ALERT

11. LCD Display

Normal

Heart:72 BPM
SpO2:98%

Alert

Heart:48 BPM
ALERT!

12. CSV Logging

File:

/data/health_log.csv

Example:

Timestamp,HeartRate,SpO2,Status
00:00:02,72,98,NORMAL
00:00:04,74,98,NORMAL
00:00:06,49,97,ALERT
00:00:08,81,92,ALERT
00:00:10,76,99,NORMAL

13. GitHub README Reference

Include the following in your README.md:

## Health Data Logs

Health monitoring logs are stored in:

data/health_log.csv

The CSV file records:

- Timestamp
- Heart Rate (BPM)
- SpO₂ (%)
- Status (NORMAL/ALERT)

14. Evaluation Criteria

Sensor Interface

- Uses MAX30102 over I²C (or LDR simulation).

Signal Processing

- Implements a 5-reading rolling average to smooth noisy measurements.

Alerts

- Red LED activates for abnormal heart rate or low SpO₂ values.

Data Logging

- Readings are displayed every 2 seconds and recorded in data/health_log.csv for analysis and GitHub documentation.

15. Conclusion

The proposed Heart Rate & SpO₂ Monitor demonstrates basic biomedical sensor interfacing using an ESP32 and a MAX30102 (or simulated LDR input). A 5-reading rolling average improves measurement stability, while threshold-based LED alerts provide immediate feedback for abnormal values. Logging the measurements to a CSV file supports later analysis and project documentation. This educational prototype helps students understand health-monitoring concepts but is not intended for medical diagnosis or clinical use.

Smart City IoT Proposal for Jammu

Introduction

The Government of Jammu & Kashmir is developing Smart City initiatives to improve urban infrastructure, public safety, and resource management. IoT (Internet of Things) enables real-time monitoring, automation, and data-driven decision-making through connected sensors and cloud platforms. This proposal focuses on three important Smart City applications for Jammu: Smart Street Lighting, Smart Parking Management, and Air Quality Monitoring.

1. Smart Street Lighting

Objective

Reduce electricity consumption while improving road safety by automatically controlling street lights based on ambient light and vehicle/pedestrian movement.

Sensors Required

- LDR (Light Dependent Resistor): Detects day/night conditions.

- PIR Motion Sensor: Detects pedestrians or vehicles.

Microcontroller & Connectivity

- ESP32

- Wi-Fi (urban deployment) or LoRaWAN (large-area deployment)

Data Flow

LDR + PIR Sensor

↓

ESP32

↓

MQTT

↓

Cloud IoT Platform

↓

Smart City Dashboard

Approximate Cost per Node

| Component | Cost (INR) |
| --- | --- |
| ESP32 | ₹500 |
| LDR | ₹30 |
| PIR Sensor | ₹120 |
| Relay Module | ₹80 |
| Power Supply & Miscellaneous | ₹250 |
| Total | ≈ ₹980 |

2. Smart Parking Management

Objective

Help drivers locate available parking spaces, reducing traffic congestion, fuel consumption, and parking search time.

Sensors Required

- HC-SR04 Ultrasonic Sensor (detects vehicle presence)

Microcontroller & Connectivity

- ESP32

- Wi-Fi or LoRaWAN

Data Flow

Ultrasonic Sensor

↓

ESP32

↓

HTTP / MQTT

↓

Cloud Database

↓

Mobile App / Dashboard

Approximate Cost per Node

| Component | Cost (INR) |
| --- | --- |
| ESP32 | ₹500 |
| HC-SR04 Sensor | ₹120 |
| LED Indicator | ₹50 |
| Power Supply & Miscellaneous | ₹250 |
| Total | ≈ ₹920 |

3. Smart Air Quality Monitoring

Objective

Continuously monitor pollution levels across Jammu to support environmental protection and public health.

Sensors Required

- MQ-135 Gas Sensor (air quality)

- DHT11 (temperature and humidity)

Microcontroller & Connectivity

- ESP32

- Wi-Fi with MQTT communication

Data Flow

MQ-135 + DHT11

↓

ESP32

↓

MQTT

↓

Cloud IoT Platform

↓

Environmental Dashboard

Approximate Cost per Node

| Component | Cost (INR) |
| --- | --- |
| ESP32 | ₹500 |
| MQ-135 Sensor | ₹250 |
| DHT11 Sensor | ₹150 |
| Power Supply & Miscellaneous | ₹250 |
| Total | ≈ ₹1,150 |

Summary of Estimated Cost

| Smart City System | Approximate Cost per Node |
| --- | --- |
| Smart Street Lighting | ₹980 |
| Smart Parking | ₹920 |
| Smart Air Quality Monitoring | ₹1,150 |

Benefits

- Reduces electricity consumption through intelligent street lighting.

- Minimizes traffic congestion by providing real-time parking availability.

- Improves environmental monitoring with continuous air quality measurements.

- Enables real-time data visualization using cloud dashboards.

- Supports predictive maintenance and informed decision-making through IoT analytics.

- Uses scalable ESP32-based sensor nodes suitable for deployment across Jammu Smart City.

Conclusion

The proposed IoT Smart City solution combines Smart Street Lighting, Smart Parking Management, and Air Quality Monitoring using ESP32-based sensor nodes connected through MQTT or HTTP protocols to cloud dashboards. Each node is affordable (approximately ₹920–₹1,150) and can be deployed at scale across Jammu. This solution will improve energy efficiency, traffic management, environmental monitoring, and overall quality of urban services while supporting the Government of Jammu & Kashmir's Smart City vision.

Q49. Security Vulnerability Analysis

Introduction

The given ESP8266 IoT program contains several common security weaknesses that could allow attackers to steal credentials, intercept data, control devices remotely, or compromise the entire IoT system. The following vulnerabilities, associated risks, and recommended solutions improve the security of the device.

Vulnerability Analysis

| Vulnerability | Risk | Recommended Solution |
| --- | --- | --- |
| 1. Hardcoded Wi-Fi SSID and Password | If the source code or firmware is leaked, attackers can obtain Wi-Fi credentials and gain unauthorized access to the network. | Store credentials in a separate config.h file, use secure provisioning, and exclude the file from Git using .gitignore. |
| 2. Using HTTP Instead of HTTPS | HTTP transmits data in plain text, allowing attackers to intercept or modify communication using man-in-the-middle attacks. | Use HTTPS with TLS/SSL (WiFiClientSecure) and verify the server certificate. |
| 3. Public MQTT Broker Without Authentication | Anyone can subscribe to or publish messages, leading to unauthorized access, data leakage, or device manipulation. | Use a private MQTT broker with username/password authentication and TLS encryption. |
| 4. No Validation of Incoming Commands | Attackers can send malicious commands that may activate relays, erase data, or damage connected equipment. | Validate all incoming commands against a predefined list before executing them. |
| 5. No Encryption for Sensor Data | Sensor readings can be intercepted or modified while in transit, compromising data integrity and privacy. | Encrypt all communications using TLS (HTTPS or secure MQTT over SSL/TLS). |
| 6. No Device Authentication | The cloud server cannot verify whether data is coming from a legitimate device, allowing spoofing attacks. | Use API keys, authentication tokens, or client certificates to authenticate devices. |

Corrected Code 1: Secure Wi-Fi Credentials

In config.h

#ifndef CONFIG_H

#define CONFIG_H

const char* WIFI_SSID = "YourWiFiName";

const char* WIFI_PASSWORD = "YourStrongPassword";

#endif

Main Program

#include <ESP8266WiFi.h>

#include "config.h"

void setup() {

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

}

}

Improvement:

- Wi-Fi credentials are separated from the main source code.

- config.h should be added to .gitignore so credentials are never uploaded to GitHub.

Corrected Code 2: HTTPS Instead of HTTP

#include <ESP8266WiFi.h>

#include <WiFiClientSecure.h>

#include <ESP8266HTTPClient.h>

WiFiClientSecure client;

HTTPClient https;

void setup() {

client.setInsecure();   // For demonstration only.

// In production, verify the server certificate.

https.begin(client, "https://example.com/api/upload");

https.GET();

https.end();

}

Improvement:

- HTTPS encrypts communication using TLS.

- Data cannot be easily intercepted or modified during transmission.

Example: Validate Incoming Commands

String command = "LED_ON";

if(command == "LED_ON"){

digitalWrite(LED_BUILTIN, LOW);

}

else if(command == "LED_OFF"){

digitalWrite(LED_BUILTIN, HIGH);

}

else{

Serial.println("Invalid Command");

}

This ensures that only authorized and expected commands are executed.

Additional Security Best Practices

- Use MQTT over TLS (port 8883) instead of unencrypted MQTT.

- Enable MQTT username/password authentication.

- Use strong, unique Wi-Fi passwords.

- Rotate API keys and credentials periodically.

- Keep ESP8266 firmware updated.

- Disable unused services and ports.

- Implement secure boot and signed firmware updates where supported.

Conclusion

The original IoT program contains several serious security vulnerabilities, including hardcoded credentials, unencrypted HTTP communication, insecure MQTT usage, lack of command validation, and missing authentication. By separating credentials into a configuration file, using HTTPS and secure MQTT, validating all incoming commands, and implementing proper authentication, the ESP8266-based IoT system becomes significantly more secure against credential theft, eavesdropping, unauthorized access, and remote attacks.

Q50. Reflection & Learning Portfolio

Throughout this IoT assignment, I explored the complete workflow of designing, programming, and understanding Internet of Things systems. Although many of the projects were developed using simulations and theoretical designs, they helped me understand how sensors, microcontrollers, cloud platforms, and communication protocols work together to solve real-world problems. I learned that IoT is much more than connecting devices to the internet—it is about collecting meaningful data, processing it intelligently, and using it to improve efficiency, safety, and decision-making.

During this assignment, I designed several IoT solutions, including a smart home air quality monitor, an automated plant watering system, a smart door lock with OTP authentication, a PIR-based security alert system, a greenhouse controller, a river flood detection system, a heart rate and SpO₂ monitoring prototype, and a smart city proposal for Jammu. Each project introduced me to different sensors such as the DHT11, MQ-2 gas sensor, HC-SR04 ultrasonic sensor, PIR motion sensor, LDR, MAX30102 pulse oximeter, and soil moisture sensor. I also gained practical knowledge of using microcontrollers like the ESP32 and ESP8266, which are widely used in modern IoT applications because of their built-in Wi-Fi and Bluetooth capabilities.

One of the most valuable lessons I learned was the importance of communication protocols such as MQTT, HTTP, HTTPS, Bluetooth, and Wi-Fi. I now understand why MQTT is preferred for IoT applications due to its lightweight publish-subscribe architecture and low bandwidth usage. I also learned that security is a critical aspect of IoT development. Hardcoding passwords, using unencrypted HTTP, and accepting unverified commands can expose devices to serious cyberattacks. Implementing HTTPS, MQTT over TLS, secure authentication, certificate validation, and proper credential management are essential practices for building secure IoT systems.

As a student of IoT and Drones, these skills will directly support my academic projects and future career. IoT devices are becoming increasingly important in agriculture, healthcare, transportation, environmental monitoring, industrial automation, and smart cities. Understanding sensor interfacing, cloud connectivity, embedded programming, and data visualization gives me a strong foundation for developing intelligent systems that solve practical problems. I also improved my problem-solving abilities by designing system architectures, selecting appropriate sensors, defining thresholds, and implementing control logic for different applications.

In the future, I would like to build an AI-powered Smart Disaster Monitoring and Emergency Response System for the Jammu region. The system would use water level sensors, rain gauges, temperature and humidity sensors, gas sensors, GPS, and cameras connected to ESP32-based sensor nodes. These nodes would communicate through LoRaWAN for long-range, low-power communication in remote areas, while gateways would upload data to the cloud using 4G/5G or Wi-Fi. An MQTT-based cloud platform would collect real-time information, and a web dashboard and mobile application would display live sensor readings, maps, alerts, and historical trends. Artificial intelligence could analyze sensor data to predict floods, landslides, or hazardous environmental conditions and automatically notify emergency services and nearby residents. Such a system could improve disaster preparedness, reduce response times, and help protect lives and property.

Overall, this assignment significantly enhanced my understanding of IoT concepts, embedded systems, communication protocols, cloud integration, and cybersecurity. It strengthened both my theoretical knowledge and practical design skills. I am now more confident in developing IoT solutions and look forward to applying these skills in future academic projects, research, and my professional career in IoT, embedded systems, and smart automation.