Smart Traffic Flow Monitoring System (ESP32 + Ultrasonic Sensor)

Components:

- ESP32

- HC-SR04 Ultrasonic Sensor

- LED Signal Simulation

Connections:

| Component | ESP32 Pin |
| --- | --- |
| Trigger | GPIO 5 |
| Echo | GPIO 18 |
| Red LED | GPIO 26 |
| Green LED | GPIO 27 |

Function:

- Counts traffic density.

- Changes signal based on traffic level.

ESP32 Code:

// Smart Traffic Monitoring System
// ESP32 + Ultrasonic Sensor

#define TRIG_PIN 5
#define ECHO_PIN 18

#define RED_LED 26
#define GREEN_LED 27


long duration;
int distance;


void setup()
{
  Serial.begin(115200);

  pinMode(TRIG_PIN, OUTPUT);
  pinMode(ECHO_PIN, INPUT);

  pinMode(RED_LED, OUTPUT);
  pinMode(GREEN_LED, OUTPUT);

  Serial.println("Smart Traffic System Started");
}



int getDistance()
{
  digitalWrite(TRIG_PIN, LOW);
  delayMicroseconds(2);

  digitalWrite(TRIG_PIN, HIGH);
  delayMicroseconds(10);

  digitalWrite(TRIG_PIN, LOW);


  duration = pulseIn(ECHO_PIN,HIGH);

  distance = duration * 0.034 / 2;

  return distance;
}



void loop()
{

 int vehicleDistance=getDistance();


 Serial.print("Vehicle Distance:");
 Serial.print(vehicleDistance);
 Serial.println(" cm");


 // Traffic density decision

 if(vehicleDistance < 50)
 {
   Serial.println("Heavy Traffic");

   digitalWrite(RED_LED,HIGH);
   digitalWrite(GREEN_LED,LOW);

 }

 else
 {

   Serial.println("Low Traffic");

   digitalWrite(RED_LED,LOW);
   digitalWrite(GREEN_LED,HIGH);

 }



 // Send data to cloud

 Serial.println("Traffic Data Uploaded");


 delay(2000);

}