Smart Waste Bin Monitoring System (ESP32 + Ultrasonic Sensor)

Components:

- ESP32

- HC-SR04 Ultrasonic Sensor

- Buzzer Alert

Connections:

| Component | ESP32 Pin |
| --- | --- |
| Trigger | GPIO 5 |
| Echo | GPIO 18 |
| Buzzer | GPIO 25 |

Function:

- Measures garbage level.

- Sends alert when bin is full.

ESP32 Code:

// Smart Waste Bin Monitoring System
// ESP32 + Ultrasonic Sensor


#define TRIG_PIN 5
#define ECHO_PIN 18
#define BUZZER 25


long duration;
int distance;


void setup()
{

Serial.begin(115200);

pinMode(TRIG_PIN,OUTPUT);
pinMode(ECHO_PIN,INPUT);

pinMode(BUZZER,OUTPUT);


Serial.println("Smart Waste Management Started");

}



int measureLevel()
{

digitalWrite(TRIG_PIN,LOW);
delayMicroseconds(2);


digitalWrite(TRIG_PIN,HIGH);
delayMicroseconds(10);


digitalWrite(TRIG_PIN,LOW);


duration=pulseIn(ECHO_PIN,HIGH);


distance=duration*0.034/2;


return distance;

}



void loop()
{

int binLevel=measureLevel();


Serial.print("Empty Space:");
Serial.print(binLevel);
Serial.println(" cm");



if(binLevel < 10)
{

Serial.println("ALERT: Garbage Bin Full");


digitalWrite(BUZZER,HIGH);

}

else
{

Serial.println("Bin Capacity Normal");

digitalWrite(BUZZER,LOW);

}



// Cloud upload simulation

Serial.println("Data Sent To Smart City Dashboard");


delay(3000);

}